Here are exam-ready notes for the given Paint Technology questions. I kept them clear and structured so they are easy to write in exams.

# 1. Metallic Pigments

Metallic pigments are pigments made from metal powders or flakes that give paints a metallic appearance and reflective effect.

### Characteristics

- Provide shiny metallic finish

- High reflectivity

- Improve decorative appearance

- Provide corrosion resistance

### Common Metallic Pigments

- Aluminium powder

  - Most commonly used metallic pigment

  - Provides silver finish

  - Used in protective coatings and decorative paints.

- Bronze powder

  - Made from copper-zinc alloys

  - Produces gold-like appearance

- Copper powder

  - Used for special decorative coatings

### Applications

- Automotive coatings

- Decorative paints

- Protective coatings

- Printing inks

# 2. Extender Pigments

Extender pigments (or fillers) are pigments added mainly to increase volume and reduce cost of paints.

### Functions

- Improve paint consistency

- Increase film thickness

- Improve durability

- Control gloss

- Reduce cost of paint

### Examples

- Calcium carbonate

- Barium sulphate

- Silica

- China clay

- Talc

### Characteristics

- Low hiding power

- Chemically inert

- Low cost

# 3. Surface Preparation Methods

Surface preparation is the process of cleaning and preparing a surface before painting to ensure proper adhesion.

### Methods

1. Mechanical Cleaning

- Wire brushing

- Sandpapering

- Grinding

2. Sand Blasting

- High pressure abrasive particles remove rust and scale.

3. Chemical Cleaning

- Use of acids or solvents to remove grease and rust.

4. Degreasing

- Removes oil and grease using solvents or alkaline solutions.

5. Pickling

- Acid treatment to remove rust and scale.

6. Phosphating

- Produces protective phosphate coating for better paint adhesion.

# 4. Hiding Power

Hiding power is the ability of a paint to completely obscure the surface underneath it.

### Factors Affecting Hiding Power

- Type of pigment

- Particle size

- Pigment concentration

- Film thickness

- Refractive index difference between pigment and binder

### Example

Titanium dioxide (TiO₂) has very high hiding power.

# 5. Volume Solids

Volume solids is the percentage of non-volatile components present in paint that remain on the surface after drying.

### Formula

Volume Solids (%) =
[
\frac{\text{Volume of solid components}}{\text{Total volume of paint}} \times 100
]

### Importance

- Determines paint film thickness

- Influences coverage area

- Affects durability of coating

# 6. PVC (Pigment Volume Concentration)

PVC is the ratio of pigment volume to total volume of non-volatile components (pigment + binder).

### Formula

[
PVC = \frac{Volume\ of\ pigment}{Volume\ of\ pigment + Volume\ of\ binder} \times 100
]

### Importance

- Controls paint properties

- Affects gloss, strength and durability

### Critical Pigment Volume Concentration (CPVC)

- The point where binder just fills the spaces between pigment particles.

# 7. Paint Filling Defects

Paint film defects occur due to improper formulation or application.

### Common Defects

1. Blistering

- Formation of bubbles due to trapped moisture.

2. Cracking

- Paint film breaks due to excessive thickness.

3. Peeling

- Paint separates from surface due to poor adhesion.

4. Sagging

- Paint flows downward forming uneven coating.

5. Chalking

- Formation of powdery surface due to weathering.

6. Wrinkling

- Uneven drying causes wrinkles in paint film.

# 8. Paint Application Methods

Paints can be applied using several techniques.

### 1. Brush Painting

- Most common and simple method.

- Used for small surfaces.

### 2. Spray Painting

- Paint sprayed using compressed air.

- Provides smooth finish.

### 3. Roller Coating

- Roller used to spread paint evenly.

### 4. Dip Coating

- Object dipped in paint tank.

### 5. Flow Coating

- Paint allowed to flow over surface.

### 6. Electrostatic Painting

- Charged paint particles attracted to surface.

✅ If you want, I can also make a “one-page revision sheet for Paint Technology (all important definitions + formulas)” which is very useful for last-minute exam revision.