Here are clear exam-ready answers for the important questions (2nd series) from Tyre Manufacturing. I wrote them in structured points so they are easy to memorize and write in exams.

# 1. Major Rubbers Used in Tyre Manufacture

The main rubbers used in tyre manufacturing are:

1. Natural Rubber (NR)

- Obtained from latex of Hevea brasiliensis.

- Excellent tensile strength, resilience and fatigue resistance.

- Used in truck tyres, carcass and tread compounds.

2. Styrene Butadiene Rubber (SBR)

- Synthetic rubber widely used in tyre treads.

- Provides good abrasion resistance and aging resistance.

- Common in passenger car tyres.

3. Butadiene Rubber (BR)

- Has excellent wear resistance and low heat build-up.

- Improves rolling resistance and durability.

4. Butyl Rubber (IIR)

- Very low air permeability.

- Used mainly in inner liners and tyre tubes.

5. Halobutyl Rubber (CIIR / BIIR)

- Chlorinated or brominated butyl rubber.

- Used for tubeless tyre inner liner due to excellent air retention.

# 2. Function of Bias Cutter in Tyre Manufacturing

A bias cutter is used to cut rubberized fabric plies at a specific angle.

Functions:

- Cuts cord fabric at a predetermined bias angle (usually 30–40°).

- Ensures correct ply orientation in bias tyres.

- Provides accurate width and length of plies.

- Helps maintain uniform tyre strength and structure.

- Prepares plies for tyre building process.

# 3. Role of Collapsible Drum in Tyre Building

A collapsible drum is used in tyre building machines.

Functions:

- Acts as the base structure for tyre assembly.

- Allows laying of inner liner, plies, beads and tread.

- Can expand to hold tyre layers tightly during building.

- After assembly, it collapses to remove the green tyre easily.

- Helps maintain correct tyre shape and dimensions.

# 4. Radial Tyre Building is a Two-Stage Process – Why?

Radial tyres are built in two stages because their structure is complex.

Stage 1 – Carcass Building

- Inner liner applied

- Body plies placed radially

- Beads positioned

- Carcass shaped

Stage 2 – Belt and Tread Application

- Steel belts added

- Cushion gum applied

- Tread rubber placed

Reasons for two stages:

- Ensures accurate placement of steel belts

- Improves dimensional stability

- Prevents distortion of carcass plies

- Ensures better tyre strength and performance

# 5. What is Meant by Green Tyre?

A green tyre is the uncured tyre obtained after tyre building but before vulcanization.

Characteristics:

- Soft and flexible

- Contains unvulcanized rubber compounds

- Has the complete tyre structure but no final strength

- Sent to curing press for vulcanization

# 6. Fibre Reinforcement Materials Used in Tyres

Fibre reinforcements provide strength, flexibility and dimensional stability.

### 1. Cotton

- One of the earliest materials used.

- Good flexibility but low strength.

- Mostly replaced by synthetic fibres.

### 2. Rayon

- Good heat resistance and dimensional stability.

- Used in carcass plies of some tyres.

### 3. Nylon (Polyamide)

- High strength and fatigue resistance.

- Good impact resistance.

- Widely used in truck tyres and bias tyres.

### 4. Polyester

- Good dimensional stability and durability.

- Used in passenger car radial tyres.

### 5. Aramid (Kevlar type fibres)

- Very high strength-to-weight ratio.

- Used in high-performance tyres.

### 6. Steel Cord

- Used in belt layers of radial tyres.

- Provides high stiffness and puncture resistance.

# 7. Manufacture of Tyre Tube (with Formulation)

### Typical Tube Compound Formulation

| Material | Parts by Weight (phr) |
| --- | --- |
| Natural Rubber | 100 |
| Carbon Black | 40–50 |
| Zinc Oxide | 5 |
| Stearic Acid | 1–2 |
| Antioxidant | 1–2 |
| Sulphur | 2–3 |
| Accelerator | 0.5–1 |

### Manufacturing Process

- Compounding

  - Raw rubber mixed with chemicals in internal mixer or two-roll mill.

- Extrusion

  - Rubber compound extruded through tube die to form tubular shape.

- Splicing

  - Ends of tube joined to form a continuous ring.

- Valve Fixing

  - Valve stem attached to tube.

- Vulcanization

  - Tube cured in steam curing mould or autoclave.

- Inspection

  - Tube checked for air leakage and defects.

# 8. Methods of Tyre Curing

Tyre curing (vulcanization) converts green tyre → finished tyre.

### 1. Mould Curing

- Green tyre placed in mould.

- Heat and pressure applied.

- Tread pattern formed.

### 2. Bag-O-Matic System

- Uses inflatable curing bladder.

- Bladder presses tyre against mould surface.

### 3. Autoclave Curing

- Tyres cured using steam pressure in autoclave.

- Used for some special tyres.

### 4. Segmented Mould Curing

- Mould consists of multiple segments.

- Provides accurate tread formation.

# 9. Steps of Passenger Car Tyre Building

Tyre building involves assembling components on a tyre building drum.

Steps:

- Inner liner application

- Carcass ply placement

- Bead placement

- Ply turn-up around beads

- Sidewall rubber application

- Belt layer placement (radial tyres)

- Cushion gum application

- Tread placement

- Tyre shaping

- Removal of green tyre

# 10. Post Cure Inflation (PCI)

### Significance

Post cure inflation is done after curing to maintain tyre shape.

Importance:

- Prevents tyre shrinkage after vulcanization

- Maintains correct tyre dimensions

- Improves uniformity and stability

- Prevents sidewall deformation

### Procedure

- Tyre removed from curing press.

- Tyre placed on PCI machine.

- Tyre inflated with air or nitrogen.

- Tyre cooled while inflated.

- Tyre retains correct shape and size.

# 11. RFL Dip Process

RFL = Resorcinol – Formaldehyde – Latex

This process improves adhesion between textile cords and rubber.

### Process Steps

- Preparation of RFL Solution

  - Resorcinol + formaldehyde resin mixed with latex.

- Dipping

  - Textile cords passed through RFL dipping bath.

- Drying

  - Excess solution removed and cords dried.

- Heat Treatment

  - Heat activates bonding layer.

- Rubber Coating

  - Treated cords used in tyre manufacturing.

### Advantages

- Strong rubber-cord bonding

- Improves tyre durability

- Prevents cord separation

✅ If you want, I can also make a “Super Short 3-mark + 5-mark exam version” of these answers so you can revise the whole tyre manufacturing chapter in 10 minutes before the exam.